# Protection concepts ATEX and IECEx

The table below lists the types of protection methods and basic concepts of protection according to ATEX and IECEx for electrical equipment that is to be operated or installed in hazardous areas. The matrix is constructed to enable the fast cross referencing of protection concepts for zones and related equipment protection level.

For definitions of hazardous zones, please see Hazardous areas according to ATEX, IECEx zones and North American Classes and Divisions section.

Similar matrix of protection concepts can also be found for the US and Canadian standards.

Protection concepts according to ATEX / IECEx

| Symbol | Type of protection | Basic concept of protection | Suitable for Zones |  |  |  |  |  | Typical EPL |  |  |  |  |  | EN / IEC Standard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 | 20 | 21 | 22 | Ga | Gb | Gc | Da | Db | Dc |  |
| e | Increased safety | No arcs, sparks or hot surfaces, enclosure IP54 or better |  | • | • |  |  |  |  | • |  |  |  |  | 60079-7 |
| n | Type 'n' (non-sparking) |  |  |  | • |  |  |  |  |  | • |  |  |  | 60079-15 |
|  | Type 'n' (closed-break) | Containment of the explosion |  |  | • |  |  |  |  |  | • |  |  |  |  |
|  | Type 'n' (sealed and hermetically sealed) | Keep the flammable substances out |  |  | • |  |  |  |  |  | • |  |  |  |  |
|  | Type 'n' (restriced breathing) |  |  |  | • |  |  |  |  |  | • |  |  |  |  |
| d | Flameproof | Containment of the explosion |  | • | • |  |  |  |  | • |  |  |  |  | 60079-1 |
| q | Powder filled | Quenching of the flame |  | • | • |  |  |  |  | • |  |  |  |  | 60079-5 |
| ia | Intrinsic safety | Limitation of spark energy and surface temperatures | • | • | • | • | • | • | • |  |  | • |  |  | 60079-11 |
| ib |  |  |  | • | • |  | • | • |  | • |  |  | • |  |  |
| ic |  |  |  |  | • |  |  | • |  |  | • |  |  | • |  |
| px | Pressurised enclosure | Keep the flammable substances out |  | • | • |  |  |  |  | • |  |  |  |  | 60079-2 |
| py |  |  |  | • | • |  |  |  |  | • |  |  |  |  |  |
| pz |  |  |  |  | • |  |  |  |  |  | • |  |  |  |  |
| ma | Encapsulation |  | • | • | • | • | • | • | • |  |  | • |  |  | 60079-18 |
| mb |  |  |  | • | • |  | • | • |  | • |  |  | • |  |  |
| mc |  |  |  |  | • |  |  | • |  | • |  |  |  |  |  |
| o | Oil immersion |  |  | • | • |  |  |  |  | • |  |  |  |  | 60079-6 |
| Op pr | Optical radiation | Inherently safe, protected by shutdown |  | • | • |  |  |  |  | • |  |  |  |  | 60079-28 |
| Op sh |  |  | • | • | • |  |  |  | • |  |  |  |  |  |  |
| Op is |  |  | • | • | • |  |  |  | • |  |  |  |  |  |  |
| ta | Dust ignition protection by enclosure | Dust-tight enclosure |  |  |  | • | • | • |  |  |  | • |  |  | 60079-31 |
| tb |  |  |  |  |  |  | • | • |  |  |  |  | • |  |  |
| tc |  |  |  |  |  |  |  | • |  |  |  |  |  | • |  |
| pd | Pressurised enclosure | Keep the flammable substances out |  |  |  |  | • | • |  |  |  |  | • |  | 61241-4 |
|  |  |  |  |  |  |  |  | • |  |  |  |  |  |  |  |